Nose Gear:

Topology Structural Test & Optimization

Material Properties

| Property | Value | Units |
| --- | --- | --- |
| Young Modulus (E) | 4.2e9 | Pa |
| Poisson’s Ratio | 0.36 | - |
| Density | 1080 | kg/m^3 |
| Yield Strength | 38 | MPa |
| Ultimate (UTS) | 47.8 | MPa |

From: https://juggerbot3d.com/pla-filament-review/#:~:text=Polylactic%20Acid%20(PLA)%20is%20a%20recyclable%2C%20natural,D%2066%20*%20**Density**%201.08%20%2D%201.12g/cc

Boundary Conditions

| Fixed Supports: Bolts to Fuselage | Force: Components (X, Y, Z) at Wheel |
| --- | --- |

Mesh Convergence

V1: Vertical – Static Parked

-XXX: Percentage of Fidelity in “Explore”, e.g., -050 = 50%

-XXXR: Percentage of Fidelity in “Refine” with Local Refinement; 1 mm and 0.5 mm

-XXXR2: Percentage of Fidelity in “Refine” with Local Refinement; 0.5 mm and 0.25 mm

-XXXR2F: Percentage of Fidelity in “Refine” with Local Refinement; 0.5 mm and 0.25 mm and 0.2mm filet in holes for stress singularity

| Case | Description | Force (N) | Max Principal Stress (Pa) | Variation (%) |
| --- | --- | --- | --- | --- |
| V1-050 | Static Parked | 4.81 | 1.89e6 | - |
| V1-058 | - | - | 2e6 |  |
| V1-066 | - | - | 1.84e6 |  |
| V1-075 | - | - | 1.76e6 |  |
| V1-083 | - | - | 2.08e6 |  |
| V1-091 | - | - | 1.93e6 |  |
| V1-100 | - | - | 2.06e6 |  |
| V1-050R | - | - | 4.2e6 |  |
| V1-075R | - | - | 3.47e6 |  |
| V1-100R | - | - | 3.95e6 |  |
| V1-100R2 |  |  | 5.86e6 |  |

Von Misses Stresses criterion was selected for failure evaluation because: PLA is not purely brittle – 25.7% elongation. VMS valid for ductile/semi-ductile materials such as PLA. Numerical stability is demonstrated in VMS (ignore last data point due to radius addition), whereas principal stress is heavily reliant of mesh due to singularity issues. Additionally, VMS is conservative is this case (worse case).

Final Mesh: 100% Fidelity in “Refine” with local refinement of 0.5 mm in B-TH and 0.25 in T-TH. Number of Elements: 78706. Number of Nodes: 123572. Element Type: Quadratic Tet (Tend to underestimate in displacement but not much compared to Linear Tet).

| Final Mesh: Side View | Top View |
| --- | --- |
| Local Sizing in T-TH | Local Sizing in B-TH |

Test Matrix

Mass of the BUSR V3: 4.9kg

Vertical (+Y):

| Case | Description | Nose % | g (m/s^2) | Force (N) |
| --- | --- | --- | --- | --- |
| V1 | Static Parked | 10 | 1 | 4.81 |
| V2 | Normal Landing (Main Gear 1st) | 10 | 3 | 14.42 |
| V3 | Hard Landing | 15 | 5 | 36.05 |
| V4 | Nose-first Landing | 50 | 5 | 120.17 |
| V5 | Nose strike / crash | 50 | 10 | 240.35 |

Lateral and Axial (X, Z):

| Gust (X) | Grass Drag (X, Z) | Stuck in Mud (Z) |
| --- | --- | --- |
| A = 1.5 m × 0.12 m = 0.18 m² F = 0.5 × 1.225 × (3)² × 1.3 × 0.18 F ≈ 1.3 N | a = 2 / 1 = 2 m/s² F = 4.9 × 2 = 9.2 N | a = 3 / 0.3 = 10 m/s² F = 5 × 10 = 49 N |

| Case | Description | Force (N) |
| --- | --- | --- |
| L1 | Gust | ±1.3 |
| L2, A2 | Grass Drag | ±9.2 |
| L3, A3 | Stuck in Mud | ±49 |

Combined:

| Case | Description | Vertical (N) | Lateral (N) |
| --- | --- | --- | --- |
| C1 | Normal Landing + Gust | 14.42 | ±1.3 (L) |
| C2 | Hard Landing + Grass Drag | 36.05 | ±9.2 (L) |
| C3 | Nose-first + Mud (Maybe too Extreme) | 120.17 | ±49 (A) |

Results

Focusing primarily on Maximum (First) Principal Stress due to brittle fracture criterion, with a limit of < 38 MPa or 3.8e7 Pa. Secondly, sanity checks such as Location of maximum stress, Von Mises; if lower, confirms multiaxial stress state

Vertical (+Y):

| Case | VMS (Pa) | Safety Factor | Location | Displacement (mm) | Pass? |
| --- | --- | --- | --- | --- | --- |
| V1 | 3.48e6 | 10.9 | T-TH | 0.439 | Yes |
| V2 | 1.04e7 | 3.64 | T-TH | 1.31 | Yes |
| V3 | 2.61e7 | 1.46 | T-TH | 3.29 | Yes |
| V4 | 8.7e7 | 0.437 | T-TH | 11 | Cooked |
| V5 | 1.74e8 | 0.219 | T-TH | 21.9 | Cooked |

Lateral (X):

| Case | VMS (Pa) | Safety Factor | Location | Displacement (mm) | Pass? |
| --- | --- | --- | --- | --- | --- |
| L1 | 2.24e6 | 17 | T-Arm | 0.563 | Yes |
| L2 | 1.59e7 | 2.4 | T-Arm | 3.99 | Yes |
| A2 | 1e7 | 3.79 | T-TH | 1.54 | Yes |
| L3 | 8.44e7 | 0.45 | T-Arm | 21.2 | Cooked |
| L3* | <2.6e7 | 1.46 | C (Bijoy) | - | Yes |
| A3 | 5.34e7 | 0.711 | T-TH | 8.2 | Cooked |
| A3* | <2.19e7 | 1.72 | C (Bijoy) | - | Yes |

| Front View: X component | Side View: Z component |
| --- | --- |

Region of Flight test Failure (Bijoy):

A3: Location of Failure @ Flight Test

L3: Location of Failure @ Flight Test

| Case | VMS (Pa) | Safety Factor | Location | Displacement (mm) | Pass? |
| --- | --- | --- | --- | --- | --- |
| C1 |  |  |  |  | Yes |
| C2 | 2.7e7 | 1.41 | T-TH, T-Arm | 2.85 | Yes |
| C3 | - | - | - | - | Cooked |